Section 1: Identification

  - Identification

Product form: Mixture Liquid

Product name: BRIJ-GUARD Polyaspartic No.85 A-Side

  - Recommended use and restrictions on use.

This product has no additional information on restriction or usage

  - Supplier

BRIJ BUILDING SOLUTION

10606 Shady Trail Suite 103

Dallas, Texas 75220

T 214-625-1600

  - Emergency telephone number

Emergency number: 214-622-1600 (Monday - Friday 7 am - 5 pm Central Time)

Section 2: Hazard(s) Identification

2.1. Classification of the substance or mixture

This material is considered hazardous by the OSHA Hazard Communication Standard

(29 CFR 1910.1200).

GHS US classification

Skin corrosion/irritation - category 1b

Serious eye damage/ eye irritation - category 1

Skin sensitization - category 1

Aquatic hazard (acute) - category 3

Aquatic hazard (long-term) - category 3

2.2. GHS Label elements, including precautionary statements.

GHS US labeling

Hazard pictograms (GHS US)

Signal word (GHS US): Danger

Hazard statements (GHS US):

H314 - Causes severe skin burns and eye damage.

H317 - May cause an allergic skin reaction.

H412 - Harmful to aquatic life with long lasting effects.

Precautionary statements (GHS US):

P280 - Wear protective gloves, protective clothing and eye or face protection.

P273 - Avoid release to the environment.

P261 - Avoid breathing vapor.

P264 - Wash thoroughly after handling.

P272 - Contaminated work clothing must not be allowed out of the workplace.

P304 + P340, P310 - IF INHALED: Remove person to fresh air and keep comfortable

for breathing. Immediately call a POISON CENTER or doctor.

P301 + P310, P330, P331 - IF SWALLOWED: Immediately call a POISON CENTER or

doctor. Rinse mouth. Do NOT induce vomiting.

P303 + P361 + P353, P310 - IF ON SKIN (or hair): Take off immediately all

contaminated clothing. Rinse skin with water. Immediately call a POISON CENTER or doctor.

P363 - Wash contaminated clothing before reuse.

P302 + P352 - IF ON SKIN: Wash with plenty of water.

P333 + P313 - If skin irritation or rash occurs: Get medical advice or attention.

P305 + P351 + P338, P310 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a POISON CENTER or doctor.

2.3. Other hazards which do not result in classification.

No additional information available

2.4. Unknown acute toxicity (GHS US)

Not applicable

Section 3: Composition and Information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % |
| --- | --- | --- |
| Aspartic Acid, N, N’-(methylenedi-4,1-cyclohexanediyl) bis-, 1,1’,4,4’-tetraethyl ester | CAS 136210-30-5 | ≥50 - ≤80 |
| cycloaliphatic diamine | CAS 54914-37-3 | ≥10 - ≤25 |
| Solvent naphtha, petroleum, light aromatic | 64742-95-6 | ≥1 - ≤5 |
| Naphtha, petroleum, heavy alkylate | 64742-95-6 | ≥1 - ≤5 |
| bis(1,2,2,6,6-Pentamethyl-4-piperidyl) sebacate | 41556-26-7 | ≥0.3 - ≤1 |

The specific chemical\ component identities and/or the exact component percentages of this material may be withheld as trade secrets. This information is made available to health professionals, employees, and designated representatives in accordance with the applicable provisions of 29 CFR 1910.1200 (I)(1). Trace ingredients (if any) are present in < 1% concentration, (< 0.1% for potential carcinogens, mutagen, and reproductive toxicant, respiratory tract, and skin sensitizers in addition to oral/ inhalation acute toxicant in category 1 and 2). None of the trace ingredients contribute significant additional hazards at the concentrations that may be present in this product. All pertinent hazard information has been provided in this document, per the requirements of the Federal Occupational Safety and Health Administration Standard (29 CFR 1910.1200), U.S. State equivalents.

Section 4: First-aid measures

4.1. Description of first aid measures

First-aid measures general : IF exposed or concerned: Get medical advice/attention. Call a poison                  center/doctor/physician if you feel unwell.

First-aid measures after inhalation : Get medical attention immediately.  Call a poison center or physician.  Remove victims to fresh air and keep at rest in a position comfortable for breathing.  If it is suspected that fumes are still present, the rescuer should wear an appropriate mask or self-contained breathing apparatus.  If not breathing, if breathing is irregular or if respiratory arrest occurs, provide artificial respiration or oxygen by trained personnel.  It may be dangerous for the person providing aid to give mouth-to-mouth resuscitation.  If unconscious, place in recovery position and get medical attention immediately.  Maintain an open airway.  Loosen tight clothing such as a collar, tie, belt or waistband.  In case of inhalation of decomposition products in a fire, symptoms may be delayed.  The exposed person may need to be kept under medical surveillance for 48 hours

First-aid measures after skin contact : Get medical attention immediately.  Call a poison center or physician.  Wash with plenty of soap and water.  Wash contaminated clothing thoroughly with water before removing it, or wear gloves.  Continue to rinse for at least 20 minutes.  Chemical burns must be treated promptly by a physician.  In the event of any complaints or symptoms, avoid further exposure.  Wash clothing before reuse.  Clean shoes thoroughly before reuse.

First-aid measures after eye contact : Get medical attention immediately.  Call a poison center or physician.  Immediately flush eyes with plenty of water, occasionally lifting the upper and lower eyelids.  Check for and remove any contact lenses.  Continue to rinse for at least 20 minutes.  Chemical burns must be treated promptly by a physician.

First-aid measures after ingestion : Get medical attention immediately.  Call a poison center or physician.  Wash out mouth with water.  Remove dentures if any.  If material has been swallowed and the exposed person is conscious, give small quantities of water to drink.  Stop if the exposed person feels sick as vomiting may be dangerous.  Do not induce vomiting unless directed to do so by medical personnel.  If vomiting occurs, the head should be kept low so that vomit does not enter the lungs.  Chemical burns must be treated promptly by a physician. Never give anything by mouth to an unconscious person.  If unconscious, place in recovery position and get medical attention immediately.  Maintain an open airway.

4.2. Most important symptoms and effects (acute and delayed)

Symptoms/effects after inhalation : May cause respiratory irritation. May cause an allergy or asthma symptoms or breathing difficulties if inhaled.

Symptoms/effects after skin contact : Irritation. May cause an allergic skin reaction.

Symptoms/effects after eye contact : Eye irritation. Causes serious eye damage.

Symptoms/effects after Ingestion: Adverse symptoms may include stomach pains

4.3. Immediate medical attention and special treatment, if necessary

| Notes to physician Specific treatments | : : | In case of inhalation of decomposition products in a fire, symptoms may be delayed. The exposed person may need to be kept under medical surveillance for 48 hours. No specific treatment. |
| --- | --- | --- |
| Protection of first aiders | : | No action shall be taken involving any personal risk or without suitable training. If it is suspected that fumes are still present, the rescuer should wear an appropriate mask or self-contained breathing apparatus. It may be dangerous to the person providing aid to give mouth-to-mouth resuscitation. Wash contaminated clothing thoroughly with water before removing it, or wear gloves. |

Section 5: Fire-Fighting Measures

5.1. Suitable (and unsuitable) extinguishing media

Use an extinguishing agent suitable for the surrounding fire

5.2. Specific hazards arising from the chemical.

This material is harmful to aquatic life with long lasting effects.  Fire water contaminated with this material must be contained and prevented from being discharged to any waterway, sewer or drain.

5.3. Special protective equipment and precautions for fire-fighters

Protection during firefighting : Do not attempt to act without suitable protective equipment. Self-contained breathing apparatus. Complete protective clothing. Promptly isolate the scene by removing all persons from the vicinity of the incident if there is a fire.  No action shall be taken involving any personal risk or without suitable training.

Section 6: Accidental Released Measures

6.1. Personal precautions, protective equipment, and emergency procedures

6.1.1. For non-emergency personnel

Emergency procedures : Ventilate spillage area. Do not breathe mist, spray, vapors. Avoid contact with skin and eyes. No action shall be taken involving any personal risk or without suitable training. Evacuate surrounding areas.  Keep unnecessary and unprotected personnel from entering.  Do not touch or walk through spilled material.  Do not breathe vapor or mist. Provide adequate ventilation.  Wear an appropriate respirator when ventilation is inadequate.  Put on appropriate personal protective equipment.

6.1.2. For emergency responders

If specialized clothing is required to deal with the spillage, take note of any information in Section 8 on suitable and unsuitable materials.  See also the information in "For nonemergency personnel".

6.2. Environmental precautions:   Avoid release to the environment.

Avoid dispersal of spilled material and runoff and contact with soil, waterways, drains and sewers.  Inform the relevant authorities if the product has caused environmental pollution (sewers, waterways, soil or air).  Water polluting material.  May be harmful to the environment if released in large quantities.

6.3. Methods and material for containment and cleaning up.

Methods for cleaning up: Take liquid spill into absorbent material. Notify authorities if product enters sewers or waters.

other information : Dispose of materials or solid residues at an authorized site.

Small spill: Stop leak if without risk.  Move containers from spill area.  Dilute with water and mop up if water-soluble.  Alternatively, or if water-insoluble, absorb with an inert dry material and place in an appropriate waste disposal container.  Dispose of via a licensed waste disposal contractor.

Large Spill: Stop leak if without risk.  Move containers from spill area.  Approach release from upwind.  Prevent entry into sewers, water courses, basements or confined areas.  Wash spillages into an effluent treatment plant or proceed as follows.  Contain and collect spillage with non-combustible, absorbent material e.g. sand, earth, vermiculite or diatomaceous earth and place in container for disposal according to local regulations (see Section 13).  Dispose of via a licensed waste disposal contractor.  Contaminated absorbent material may pose the same hazard as the spilled product.  Note: see Section 1 for emergency contact information and Section 13 for waste disposal.

6.4. Reference to other sections : For further information refer to section 13.

Section 7: Handling and Storage

7.1. Precautions for safe handling

Precautions for safe handling:

Obtain special instructions before use. Do not handle until all safety precautions have been read and understood. Wear personal protective equipment. Do not breathe mist, spray, vapors. Use only outdoors or in a well-ventilated area. Avoid contact with skin and eyes.

Hygiene measures:

Wash contaminated clothing before reuse. Contaminated work clothing should not be allowed out of the workplace. Do not eat, drink, or smoke when using this product. Always wash hands after handling the product.

7.2. Conditions for safe storage, including any incompatibilities.

Store between the following temperatures: 15 to 35°C (59 to 95°F). Store in accordance with local regulations. Store in original container protected from direct sunlight in a dry, cool and well-ventilated area, away from incompatible materials (see Section 10) and food and drink. Store locked up. Keep container tightly closed and sealed until ready for use. Containers that have been opened must be carefully resealed and kept upright to prevent leakage. Do not store in unlabeled containers. Use appropriate containment to avoid environmental contamination. See Section 10 for incompatible materials before handling or use

Section 8: Exposure Controls and Personal Protection

8.1. Control parameters

| Ingredient name | Exposure limits |
| --- | --- |
| Aspartic Acid, N, N’-(methylenedi-4,1- cyclohexanediyl)bis-, 1,1’,4,4’-tetraethyl ester | None. |
| TetraethylN,N’-(methylenedicyclohexane-4,1-diyl)bis-dl-aspartate | None. |
| 1,3,3-Trimethyl-N-(2-methylpropylidene)-5-[(2-methylpropylidene)amino] | None. |
| cyclohexane methylamine Diethyl fumarate | None. |
| Naphtha (petroleum), heavy alkylate | None. |
| bis(1,2,2,6,6-Pentamethyl-4-piperidyl) sebacate | None. |
| Methyl 1,2,2,6,6-pentamethyl-4-piperidyl sebacate | None. |

8.2. Appropriate engineering controls

Appropriate engineering controls : If user operations produce dust, fumes, gas, vapor or mist, use process enclosures, local exhaust ventilation or other engineering controls to keep worker exposure to airborne contaminants below any recommended or statutory limits.

Environmental exposure controls : Avoid release to the environment. Emissions from ventilation or work process equipment should be checked to ensure they comply with the requirements of environmental protection legislation.

8.3. Individual protection measures/Personal protective equipment

Hand protection

Protective gloves

Eye protection

Protective gloves

Skin and Body Protection

Wear suitable protective clothing

Eye protection

In case of insufficient ventilation, wear suitable respiratory equipment

Personal protective equipment symbol(s):

Section 9: Physical and chemical properties

| Physical State | : Liquid |
| --- | --- |
| Appearance | : Gel |
| Color | : Clear to Light yellow |
| Odor | : Faint Aromatic, Earthy, Musty Odor |
| Odor Threshold | : No Data available |
| pH | : 7 |
| Melting Point | : Not Applicable |
| Freezing Point | : Data Not Available |
| Boiling Point | : 302°C (575.6°F) |
| Flash Point | : 204°C (399.2°F) |
| Relative evaporation rate | : (butyl acetate=1) Data not available |
| Flammability | : Not Applicable |
| Vapor Pressure | : No Data Available |
| Relative Vapor Density at 20C | : No Data Available |
| Relative Density | : 9.61 lb./gal |
| Density | : 1.16 g/ml |
| Solubility | : Not Applicable |
| Partition coefficient n-octanol/water (Log Pow) | : No Data Available |
| Auto-ignition temperature | : No Data Available |
| Decomposition temperature | : No Data Available |
| Viscosity, kinematic | : No Data Available |
| Viscosity, dynamic | : 150-200 Cp |
| Explosion limits | : No Data Available |
| Explosive properties | : Not Classified |

Section 10: Stability and Reactivity

10.1. Reactivity

The product is non-reactive under normal conditions of use, storage, and transport. Keep away from any contact with water, because of violent reaction and flash fire.

10.2. Chemical stability

Stable under normal conditions.

10.3. Possibility of hazardous reactions

No dangerous reactions known under normal conditions of use.

10.4. Conditions to avoid.

High temperatures should be avoided

10.5. Incompatible materials

Incompatible with water, humid air. Strong bases and oxidizers

10.6. Hazardous decomposition products

Under normal conditions of storage and use, hazardous decomposition products should not be produced.

Section 11: Toxicological information

| Acute Toxicity |  |
| --- | --- |
| Diethyl fumarate | 1780 mg/kg |
| Skin Corrosion/Irritation | : Not Classified |
| Serious eye damage/irritation | : Causes serious eye damage |
| Respiratory or skin sensitization | : May cause an allergy or asthma symptoms or breathing difficulties if inhaled. May cause an allergic skin reaction. |
| Germ Cell mutagenicity | : Not Classified |
| Carcinogenicity | : Not Classified |
| Aspiration Hazard | : Naphtha (petroleum), heavy alkylate: Category 1 |
| Viscosity, Kinematic | : Data not available |
| Symptoms/effects after inhalation | No known significant effects or critical hazards |
| Symptoms/effects after skin contact | Adverse symptoms may include the following: pain or irritation redness blistering may occur |
| Symptoms/effects after eye contact | Adverse symptoms may include the following: pain watering redness |

Section 12: Ecological information

12.1. Toxicity

| Diethyl fumarate |  |
| --- | --- |
| LC50 - Fish [1] | 4500 µg/L Fresh water |

12.2. Persistence and degradability

No additional information available

12.3. Bio accumulative potential

No additional information available

| bis(4-(1,2-bis(Ethoxycarbonyl) -3-methylcyclohexyl)methane |  |
| --- | --- |
| 5.99 (Log Pow) | 10.46 Source: Quantitative Structure Activity Relation |
| TetraethylN,N’- (methylenedicyclohexane4,1-diyl)bis-dl-aspartate |  |
| 5.16 (Log Pow) | 4.51 Source: ECHA |

12.4. Mobility in soil

No additional information available

12.5. Other adverse effects

No additional information available

Section 13: Disposal Consideration

13.1. Disposal methods Waste treatment methods.

The generation of waste should be avoided or minimized wherever possible. Disposal of this product, solutions and any by-products should comply with the requirements of environmental protection and waste disposal legislation and any regional local authority requirements. Dispose of surplus and non-recyclable products via a licensed waste disposal contractor. Waste should not be disposed of untreated to the sewer unless fully compliant with the requirements of all authorities with jurisdiction. Waste packaging should be recycled. Incineration or landfill should only be considered when recycling is not feasible. This material and its container must be disposed of in a safe way. Care should be taken when handling empty containers that have not been cleaned or rinsed out. Empty containers or liners may retain some product residues. Avoid dispersal of spilled material and runoff and contact with soil, waterways, drains and sewers.

Section 14: Transportation Information

14.1. UN number : UN2735

14.2. UN proper shipping name

Proper Shipping Name (DOT) :

AMINES, LIQUID, CORROSIVE, N.O.S. (1,3,3-Trimethyl-N- (2-methylpropylidene)-5-[ (2-methylpropylidene)amino] cyclohexanemethylamine)

Proper Shipping Name (IMDG) :

AMINES, LIQUID, CORROSIVE, N.O.S. (1,3,3-Trimethyl-N- (2-methylpropylidene)-5-[ (2-methylpropylidene)amino] cyclohexanemethylamine)

Proper Shipping Name (IATA) :

AMINES, LIQUID, CORROSIVE, N.O.S. (1,3,3-Trimethyl-N- (2-methylpropylidene)-5-[ (2-methylpropylidene)amino] cyclohexanemethylamine)

4.3. Transport hazard class(es)

DOT Transport hazard class(es) (DOT) : 8

IMDG Transport hazard class(es) (IMDG) : 8

IATA Transport hazard class(es) (IATA) : 8

14.4. Packing group

Packing group (DOT) : III

Packing group (IMDG) : III

Packing group (IATA) : III

14.5. Environmental hazards

Other information : No

14.6. Special precautions for user

Transport within the user’s premises: always transport in closed containers that are upright and secure. Ensure that the persons transporting the product know what to do in the event of an accident or spillage.

14.7. Transport in bulk according to IMO instruments: Not available

Section 15: Regulatory Information

15.1. US Federal regulations

| All components of this product are present and listed as Active on the United States Environmental Protection Agency Toxic Substances Control Act (TSCA) inventory, except for: |  |  |
| --- | --- | --- |
| Aspartic Acid, N, N’-(methylenedi-4,1-cyclohexanediyl) bis-, 1,1’,4,4’-tetraethyl ester | CAS-No. 136210-30-5 CAS 136210-32-7 | 70 – 80% |
| Cycloaliphatic diamine | CAS-No. 623-91-6 | 1 – 5% |
| bis(4-(1,2-bis(ethoxycarbonyl)ethylamino)-3- methylcyclohexyl)methane | CAS-No. 136210-32-7 | 20 – 30% |
| Bis(1,2,2,6,6-pentamethyl-4- piperidyl) sebacate and Methyl 1,2,2,6,6-pentamethyl4-piperidyl sebacate | CAS-No. 1065336-91-5 | 0.1 – 0.5% |

This product or mixture is not known to contain a toxic chemical or chemicals more than the applicable de minimis concentration as specified in 40 CFR §372.38(a) subject to the reporting requirements of section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR Part 372.

15.2. International regulations:       No additional information available

15.3. US State regulations

California Proposition 65 - This product does not contain any substances known to the state of California to cause cancer, developmental and/or reproductive harm

Section 16: Other Information

US-SDS according to the federal final rule of hazard communication revised on 2012 (HazCom 2012) Safety Data Sheet (SDS), USA

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.